Baseline Summary B1238 ( to )

| Processed: | Monday, Mar 22, 2004 08:43:30AM |
| --- | --- |
| Solution type: | Iono free fixed |
| Solution acceptability: | Solution acceptable |
| Ephemeris used: | Broadcast |
| Met Data: | Standard |
| Baseline slope distance: |  |
| Elevation mask: | 15 degrees |
| Variance ratio: | 24.9 |
| Reference variance: | 0.905 |
| RMS: | 0.010m |
| Horizontal Precision 1-sigma (scaled): | 0.001m |
| Vertical Precision 1-sigma (scaled): | 0.003m |

| Start time (GPS Time): | 04/03/18, 21:42:30.000 | 1262, 423750.000 |
| --- | --- | --- |
| Stop time (GPS Time): | 04/03/18, 22:27:00.000 | 1262, 426420.000 |
| Occupation time: | 00:44:30.000 |  |

Baseline Components (Mark to Mark)

| From: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | 727127.275m | Latitude | 39°42'45.70001"N | Latitude | 39°42'45.70000"N |
| Easting | 2003122.674m | Longitude | 121°57'48.90000"W | Longitude | 121°57'48.90000"W |
| Elevation | 43.347m | Height | 15.561m | Height | 15.561m |

| To: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Grid |  | Local |  | WGS 84 |  |
| Northing | ? | Latitude | ? | Latitude | ? |
| Easting | ? | Longitude | ? | Longitude | ? |
| Elevation | ? | Height | ? | Height | ? |

| Baseline: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | ? | NS Fwd Azimuth | ? |  X | ? |
|  Easting | ? | Ell. Distance | ? |  Y | ? |
|  Elevation | ? |  Height | ? |  Z | ? |

Standard Errors

| Baseline Errors: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  Northing | 0.001m |  NS Fwd Azimuth | ? seconds |  X | 0.002m |
|  Easting | 0.001m |  Ell.Distance | ? |  Y | 0.002m |
|  Elevation | 0.003m |  Height | 0.003m |  Z | 0.003m |

Aposteriori Covariance Matrix

|  | X | Y | Z |
| --- | --- | --- | --- |
| X | 2.280e-6m2 |  |  |
| Y | 2.587e-6m2 | 4.553e-6m2 |  |
| Z | -3.231e-6m2 | -4.044e-6m2 | 7.859e-6m2 |

Occupations

|  |  | From | To |
| --- | --- | --- | --- |
| Point Name: |  |  |  |
| Data file: |  |  |  |
| Receiver Type: |  | 4000SSi | 4700 |
| Receiver Serial Number: |  | 6782 | 220202606 |
| Antenna type: |  |  |  |
| Measured To: |  |  |  |
| Antenna height | Measured | 0.000m | 0.000m |
|  | APC | 0.000m | 0.000m |

Tracking Summary

No Combined Summary available

Residuals

No Residuals Available

Processing Style - Precise Ephemeris

| Elevation Mask | 15 degrees |
| --- | --- |
| Ephemeris | Precise |
| Solution Type | Fixed |

Static

| Minimum baseline observation time | 120 seconds |
| --- | --- |
| Maximum baseline length to attempt a fixed solution : |  |
| Using broadcast ephemeris | 200 kilometers |
| Using precise ephemeris | 2000 kilometers |

Kinematic

| Minimum reference observation time | 600 seconds |
| --- | --- |
| Minimum static initialization observation time | 120 seconds |
| Minimum known point initialization ratio | 3.000 |
| Minimum OTF processing time | 200 seconds |

Global

| Frequency type | L1 |
| --- | --- |
| Maximum fixable cycle slip (static processing only) | 600 seconds |
| Maximum iterations (static processing only) | 10 |
| Maximum integer search time (static processing only) | 30 minutes |
| Antenna Model | NGS |

Quality

|  | Single frequency |  | Dual frequency |  |
| --- | --- | --- | --- | --- |
|  | Flag | Fail | Flag | Fail |
| RMS acceptance criteria | 0.030 | 0.040 | 0.020 | 0.030 |
| Ratio acceptance criteria | 3.000 | 1.500 | 3.000 | 1.500 |
| Reference variance acceptance criteria | 10.000 | 20.000 | 5.000 | 10.000 |
| Edit multiplier | 3.500 |  |  |  |

Tropo

| Model | Hopfield |
| --- | --- |
| Minimum zenith delay interval | 2 hours |
| Use observed met data | Enabled |

Iono

| Ambiguity resolution pass (static processing only) | Enabled |
| --- | --- |
| Apply to all baselines longer than | 10 kilometers |
| Final pass | Enabled |
| Apply to all baselines longer than | 5 kilometers |

Events

| Interpolation method | Linear |
| --- | --- |
| Number of points to fit | 2 |
| Maximum allowed missing epochs | 0 |
| Time offset | 0 microseconds |

OTF Search

| Search method | Optimal |
| --- | --- |