DESIGNATION  -  K 962 reset 2000

PID  - none (ref: old PID GU1669)

N 36 08 11.7

# W 120 20 45.5

Recovered: 7-29-2003

Additional recovery notes:

Station is located on Hwy 33 bridge (Bridge #42-0424) over Wartham Creek on the east side of Coalinga.  To reach from intersection of Hwy 33, Hwy 198 and W. Polk St in Coalinga go east on Hwy 33, one mile to the bridge and the station on the right.  To reach from the I-5 and Jayne Ave intersection go west 10.5 miles to the bridge and the station on the left.

Station is a NGS brass disk located at the east bridge abutment on the south side. Station is 23.5 ft. south of C/L Hwy 33, 4.0 ft. south of the south face of curb, and 0.8 ft. higher than the road surface.

NO PICTURES.